Chapter 6 Sinclair and the ULA

Sinclair's first home computer, the ZX80, was launched as a kit in February 1980 at a price of just £79.99, equivalent to £227.97 in 2010. To meet this price target its designer Jim Westwood used just 17 off-the-shelf logic chips, 1K of RAM and 4K of ROM, which contained the BASIC interpreter written by Nine Tiles Information Handling. It was simple in design and made clever use of specific Z80 instruction fetch cycles to maintain the output of the television picture.

There was little improvement that could be made to the ZX80 hardware within the £79.99 price tag, so with Sinclair's decision to produce a more advanced follow-up machine, came the need to reduce production costs.

The First Sinclair ULA

The design of the ZX80's successor, the ZX81, started in the September before the ZX80 was released. The cost associated with the proposed ZX81 's larger and more functional ROM, again from Nine Tiles, and its increased hardware complexity meant it would not be commercially viable and meet Clive Sinclair's strict price point unless the chip count could be minimised to reduce costs. By this time Ferranti and their CML ULA were well established in the semi-custom device market, and being UK based were ideally placed to provided Sinclair the means with which to achieve its goal.

Westwood took what was basically a modification of the ZX80 and turned this into a Ferranti ULA 2000 series logic design, which he wire-wrapped into a prototype but had little success in getting to work. In frustration, Westwood was forced to leave it in the hands of the new recruit, Richard Altwasser, while he took a week away from the office on business. Altwasser says he doesn't know who was more surprised when Westwood returned and unexpectedly found him in possession of a working prototype.

From this design Westwood created an interconnect layer for the Ferranti 2C000 ULA, producing the 2C184E and later the 2C210E. Around this he added the same Z80A as the ZX80, the larger 8K of ROM and again 1K RAM, creating a machine with just four ICs.

This drastic reduction in chip count significantly reduced the power consumption that was exhibited in comparison with the ZX80, which reduced the heat output and made the computer much more stable in operation. The lower chip count and manufacturing costs ultimately meant that in 1981, Sinclair could sell the ZX81 in kit form for just £49.99, equivalent to £134.47 in 2010.

With the Ferranti ULA at hand through which to realise their future technical designs, Sinclair had set the stage for the grand entrance of the ZX Spectrum.

Figure 6-1: Sinclair ZX81 2C210E ULA

The ZX Spectrum ULA

Having completed the ZX81, Westwood's experience with television saw him move on to television research and development, leaving the clearly capable Altwasser to head development of the ZX81's successor, the ZX82, as the ZX Spectrum was originally called. Work on the specification for the ZX Spectrum began in September 1981, and was mostly compiled from internal dis-

cussions between Altwasser, Westwood and Nine Tiles. There was little question that it should inherit much from the ZX81, notably the TV UHF output and reuse of code from the ZX81 ROM. This meant employing the same Z80 processor, which was also a good cost choice. The design of the ZX Spectrum was further constrained by Clive Sinclair's desire to launch the machine at the IPC Computer Fair at Earls Court on 23 April 1982, as well as his customarily low price target.

Altwasser wrote and agreed the technical specification quite quickly, and very little changed during subsequent development. One of the requirements of the design was for it to feature sound and colour, and it was television text standards that influenced the decision to use a single colour attribute per character which, as a design benefit, reduced the amount of memory required. Clive Sinclair took the decision to use Ferranti CML, having been convinced that this was a superior technical decision for the ZX81, and strongly guided Rick Dickinson's case designs. Overall, cost drove the choice of CML ULA and the memory chips used, and the need for minimal silicon real estate.

Having visited Ferranti to understand the CML ULA technology and its constraints, Altwasser produced the logic design entirely on paper and prototyped it using wire wrapped SSI 74S and 74LS TTL logic chips, all in just a few weeks. The logic design required a good understanding of the capacity and analogue capabilities of the chosen 5000 series ULA, since very few gates would remain unused, and the analogue video output made use of peripheral cells intended by Ferranti for digital interfacing. Compromise had to be made throughout as the ULA did not have the gate capacity to realise a fully synchronous design, and the interface signals had to be kept to a minimum by reducing functionality or through multiplexing to keep the total pin count from exceeding 40. The design made extensive use of the Ferranti component library, providing many of the necessary building blocks such as flip-flops and TTL outputs. Critical paths were identified by design analysis and considering gate loading, and mainly focused on memory access timing and the video output signals. Altwasser defends the lack of computer simulation, arguing that the simplicity of the design meant that the analysis and understanding of all the critical paths was within the grasp of one engineer.

When the logic design and prototype were complete, the placement of functional units within the ULA and layout of the interconnection layer on mylar film was carried out by Ferranti engineers at their offices in Oldham, Manchester. This was done jointly with Altwasser, who occasionally returned to Oldham to make layout decisions, and considered pin-out requirements that simplified PCB layout and minimised high speed signal tracks. Once the intercon-

nection layout was complete, critical path analysis was repeated. Altwasser notes that although all done manually, the steps were entirely in keeping with modern day automated post-routing simulation. He and the Ferranti engineer would study the mylar film and measure track lengths for the critical signals, calculate parasitic capacitances and the resulting signal delays and slew rates, and manually plot timing curves to ensure they were within limits. Where necessary, Altwasser would request track routing changes or even change the logic, perhaps adding buffers to meet the timing constraints.

Altwasser used both the clock and varying amounts of propagation delay to establish the desired signal timings. Because the propagation delay of a matrix cell logic gate is predictable and tunable within a range, he was able to delay and stretch signal pulses with far less complexity than by using synchronous flip-flops or counters.

Having finalised the interconnect layout, it was optically reduced to a 10x magnification light-field mask and used to create a complete multi-chip wafer mask using an optical step-and-repeat procedure, as discussed in the section called Generate Interconnect Pattern in Chapter 5, The Ferranti ULA. From this Ferranti produced an initial batch of prototype wafers and invited Altwasser to functionally test and verify the chips in-situ, before they were diced and packaged.

Altwasser was provided with a wafer probe that allowed connection to individual bond pads and made it possible to attach external circuitry with which to test the devices. Altwasser and Ferranti were under extreme time pressure, and carrying out tests during this stage saved a few precious days. However, while performing these tests and visually inspecting the interconnection tracks under a microscope, it was discovered that the Ferranti layout engineer had made an error in the interconnect layer whereby the clock output from the early counter stages was not connected to the later stages. Fortunately, and against all odds, a tiny fleck of dust had fallen onto one of the devices of the multi-chip wafer mask at exactly the point at which the missing interconnect should have been, despite the usual clean room conditions of a semiconductor plant. This artificial bridge prevented etching of the aluminium at that point, connecting the clock to the later counter stages and allowing Altwasser to complete his full test suite on this one die, successfully proving the entire chip design of the 5C102E ZX Spectrum ULA.

The first two issues of the ULA were based on the largest version of the array used for the ZX81, which provided twice the number of gates. The later issues of the ULA used a new, and larger still, 6000 series array, which in the main provided a reduced power consumption.