ТАБЛИЦА     СИМВОЛОВ

| д | Ш | Символ ZX Spectrum | Мнемоника микропроцессoра Z80 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | без префикса | с префиксом #СВ | с префиксом #ED |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 116 119 120 121 122 123 124 125 126 127 128 129 130 131 132 133 134 135 136 137 138 139 140 141 142 143 144 145 146 147 148 149 150 151 152 153 154 155 156 157 158 159 160 161 162 163 164 165 166 167 168 169 170 171 172 173 174 175 176 177 178 179 180 181 182 183 184 185 186 187 188 189 190 191 192 193 194 195 196 197 198 199 200 201 202 203 204 205 206 207 208 209 210 211 212 213 214 215 216 217 218 219 220 221 222 223 224 225 226 227 228 229 230 231 232 233 234 235 236 237 238 239 240 241 242 243 244 245 246 247 248 249 250 251 252 253 254 255 | 00 01 02 03 04 05 06 07 08 09 ОА 0В 0С 0D 0Е 0F 10 11 12 13 14 15 16 17 18 19 1А 1В 1C 1D 1Е 1F 20 21 22 23 24 25 26 27 28 29 2А 2В 2С 2D 2Е 2F 30 31 32 33 34 35 36 37 38 39 ЗА 3В ЗС 3D ЗЕ 3F 40 41 42 43 44 45 46 47 48 49 4А 4В 4С 4D 4Е 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F 70 71 72 73 74 75 76 77 78 79 7A 7B 7C 7D 7E 7F 80 81 82 83 84 85 86 87 88 89 8A 8B 8C 8D 8E 8F 90 91 92 93 94 95 96 97 98 99 9A 9B 9C 9D 9E 9F А0A1 А2 A3 А4 A5 А6 А7 А8 A9 AA AB AC AD AE AF BO B1 B2 B3 B4 B5 B6 B7 B8 B9 BA BB BC BD BE BF CO C1 C2 C3 C4 C5 C6 C7 C8 C9 CA СB СС СD СЕ CF D0 D1 D2 D3 D4 D5 D6 D7 D8 D9 DA DB DC DD DE DF EO El E2 E3 E4 E5 E6 E7 E8 E9 EA E8 EC ED ЕЕ EF FO F1 F2 F3 F4 F5 F6 F7 F8 F9 FA FB FC FD FE FF | -------------- см. Контрольне коды ZX Spectrum -------------- space ! “ # $ % & ‘ ( ) * + , - . / 0 1 2 3 4 5 6 7 8 9 : ; < = > ? @ А В С D Е F G Н I J К L М N О P Q R S Т U V W X Y Z [ / ]  _ £ a b c d e f g h i j k l m n o p q r s t u v w x y z { ¦ } ~ ©                ________________ [A] [B] [C] [D] [E] символы [F] oпределяемые [G] пользователем) [H] [I] [J] [K] [L] (символы [M] oпределяемые [N] пользователем) [O] [P] [Q] [R] [S] [Т] [U]_____________ RND INKEY$ PI FN POINT SCREEN$ ATTR AT TAB VAL$ CODE VAL LEN SIN COS TAN ASN ACS ATN LN EXP INT SQR SGN ABS PEEK IN USR STR$ CHR$ NOT BIN OR AND <= >= <> LINE THEN TO STEP DEF FN CAT FORMAT MOVE ERASE OPEN # CLOSE # MERGE VERIFY BEEP CIRCLE INK PAPER FLASH BRIGHT INVERSE OVER OUT LPRINT LLIST STOP READ DATA RESTORE NEW BORDER CONTINUE DIM REM FOR GO TO GO SUB INPUT LOAD LIST LET PAUSE NEXT POKE PRINT PLOT RUN SAVE RANDOMIZE IF CLS DRAW CLEAR RETURN COPY | NOP LD BC,nn LD (BC),A INC BC INC В DEC В LD B,n RLCA ЕХ AF,AF' ADD HL,BC LD A,(BC) DEC BC INC С DEC С LD C,n RRCA DJNZ $+e LD DE,nn LD (DE),A INC DE INC D DEC D LD D,n RLA JR $+e ADD HL,DE LD A,(DE) DEC DE INC E DEC E LD E,n RRA JR NZ,$+e LD HL,nn LD (nn),HL INC HL INC H DEC H LD H,n DAA JR Z,$+e ADD HL,HL LD HL,(nn) DEC HL INC L DEC L LD L,n CPL JR NC,$+e LD SP,nn LD (nn),A INC SP INC (HL) DEC (HL) LD (HL),n SCF JR C,$+e ADD HL,SP LD A,(nn) DEC SP INC A DEC A LD A,n CCF LD B,B LD B,C LD B,D LD B,E LD B,H LD B,L LD B,(HL) LD B,A LD C,B LD C,C LD C,D LD C,E LD C,H LD C,L LD C,(HL) LD C,A LD D,B LD D,C LD D,D LD D,E LD D,H LD D,L LD D,(HL) LD D,A LD E,B LD E,C LD E,D LD E,E LD E,H LD E,L LD E,(HL) LD E,A LD H,B LD H,C LD H,D LD H,E LD H,H LD H,L LD H,(HL) LD H,A LD L,B LD L,C LD L,D LD L,E LD L,H LD L,L LD L,(HL) LD L,A LD (HL),B LD (HL),C LD (HL),D LD (HL),E LD (HL),H LD (HL),L HALT LD (HL),A LD A,B LD А,С LD A,D LD А,Е LD А,Н LD A,L LD A,(HL) LD A,A ADD A,B ADD A,C ADD A,D ADD A,E ADD A,H ADD A,L ADD A,(HL) ADD A,A ADC A,B ADC A,C ADC A,D ADC A,E ADC A,H ADC A,L ADC A,(HL) ADC A,A SUB В SUB С SUB D SUB E SUB H SUB L SUB (HL) SUB A SBC A,B SBC A,C SBC A,D SBC A,E SBC A,H SBC A,L SBC A,(HL) SBC A,A AND В AND С AND D AND E AND H AND L AND (HL) AND A XOR В XOR С XOR D XOR E XOR H XOR L XOR (HL) XOR A OR B OR С OR D OR E OR H OR L OR (HL) OR A CP В CP C CP D CP E CP H CP L CP (HL) CP A RET NZ POP BC JP NZ,nn JP nn CALL NZ,nn PUSH BC ADD A,n RST 0 RET Z RET JP Z,nn префикс #СВ CALL Z,nn CALL nn ADC A,n RST 8 RET NC POP DE JP NC,nn OUT (N),A CALL NC,nn PUSH DE SUB n RST 16 RET С EXX JP C,nn IN A,(N) CALL C,nn префикс для IX SBC A,n RST 24 RET PO POP HL JP PO,nn ЕХ (SP),HL CALL PO,nn PUSH HL AND n RST 32 RET PE JP (HL) JP PE,nn ЕХ DE,HL CALL PE,nn префикс #ED XOR n RST 40 RET P POP AF JP P,nn DI CALL P,nn PUSH AF OR n RST 48 RET M LD SP,HL JP M,nn EI CALL M,nn префикс для IY CP n RST 56 | RLC В RLC C RLC D RLC E RLC H RLC L RLC (HL) RLC A RRC В RRC С RRC D RRC E RRC H RRC L RRC (HL) RRC A RL В RL С RLD RL E RL H RL L RL (HL) RL A RR В RR С RR D RR E RR H RR L RR (HL) RR A SLA В SLA С SLA D SLA E SLA H SLA L SLA (HL) SLA А SRA B SRA С SRA D SRA E SRA H SRA L SRA (HL) SRA A [SLI B] [SLI C] [SLI D] [SLI E] [SLI H] [SLI L] [SLI (HL)] [SLI A] SRL B SRL C SRL D SRL E SRL H SRL L SRL (HL) SRL A BIT 0,B BIT 0,C BIT 0,D BIT 0,E BIT 0,H BIT 0,L BIT 0,(HL) BIT 0,A BIT 1,B BIT 1,C BIT 1,D BIT 1,E BIT 1,H BIT 1,L BIT 1,(HL) BIT 1,A BIT 2,B BIT 2,C BIT 2,D BIT 2,E BIT 2,H BIТ 2,L BIT 2,(HL) BIT2,A BIT 3,B BIT 3,C BIT 3,D BIT 3,E BIT 3,H BIT 3,L BIT 3,(HL) BIT Э,А BIT 4,8 BIT 4,C BIT 4,D BIT 4,E BIT 4,H BIT 4,L BIT 4,(HL) BIT 4,A BIT 5,B ВIT 5,С BIT 5,D BIT 5,E BIT 5,H BIT 5,L BIT 5,(HI) BIT 5,A BIT 6,B BIT 6,C BIT 6,D BIT 6,E BIT 6,H BIT 6,L BIT 6,(HL) BIT 6,A BIT 7,B BIT 7,C BIT 7,D BIT 7,E BIT 7,H BIT 7,L BIT 7,(HL) BIT 7,A RES 0,B RES 0,C RES 0,D RES 0,E RES 0,H RES 0,L RES 0,(HL) RES 0,A RES 1,B RES ,C RES ,D RES ,E RES ,H RES ,L RES ,(HL) RES 1,A RES 2,B RES 2,C RES 2,D RES 2,E RES 2,H RES 2,L RES 2,(HL) RES 2,A RES 3,B RES 3,C RES 3,D RES 3,E RES 3,H RES 3,L RES 3,(HL) RES 3,A RES 4,B RES 4,C RES 4,D RES 4,E RES 4,H RES 4,L RES 4,(HL) RES 4,A RES 5,В RES 5,C RES 5,D RES 5,E RES 5,H RES 5,L RES 5,(HL) RES 5,A RES 6,B RES 6,C RES 6,D RES 6,E RES 6,H RES 6,L RES 6,(HL) RES 6,A RES 7,B RES 7,C RES 7,D RES 7,E RES 7,H RES 7,L RES 7,(HL) RES 7,A SET 0,B SET 0,C SET 0,D SET 0,E SET 0,H SET 0,L SET 0,(HL) SET 0,A SET 1,B SET 1,C SET 1,D SET 1,E SET 1,H SET 1,L SET 1,(HL) SET 1,A SET 2,B SET 2,C SET 2,D SET 2,E SET 2,H SET 2,L SET 2,(HL) SET 2,A SET 3,B SET 3,C SET 3,D SET 3,E SET 3,H SET 3,L SET 3,(HL) SET 3,A SET 4,B SET 4,C SET 4,D SET 4,E SET 4,H SET 4,L SET 4,(HL) SET 4,A SET 5,B SET 5,C SET 5,D SET 5,E SET 5,H SET 5,L SET 5,(HL) SET 5,A SET 6,B SET 6,C SET 6,D SET 6,E SET 6,H SET 6,L SET 6,(HL) SET 6,A SET 7,B SET 7,C SET 7,D SET 7,E SET 7,H SET 7,L SET 7,(HL) SET 7,A | IN B,(C) OUT (C),B SBC HL,BC LD (nn),BC NEG RETN IM 0 LD I,A IN C,(C) OUT (C),C ADC HL,BC LD BC,(nn) RETI LD R,A IN D,(C) OUT (C),D SBC HL,DE LD (nn),DE IM1 LD A,I IN E,(C) OUT (C),E ADC HL,DE LD DE,(nn) IM 2 LD A,R IN H,(C) OUT (C),H SBC HL,HL LD (nn),HL RRD IN L,(C) OUT (C),L ADC HL,HL LD HL,(nn) RLD IN (HL),(C) SBC HL,SP LD (nn),SP IN A,(C) OUT (C),A ADC HL,SP LD SP,(nn) LDI CPI INI OUTI LDD CPD IND OUTD LDIR CPIR INIR OTIR LDDR CPDR INDR OTDR |

Построение кодов команд Z80.

| Исходная команда |  | Получаемая команда |  |
| --- | --- | --- | --- |
| Мнемоника | Код | Мнемоника | Код |
| ADD HL,BC | #09 | ADD IX,BC | #DD #09 |
| JP (HL) | #E9 | JP (IY) | #FD #E9 |
| LD B,(HL) | #46 | LD B,(IX+1) | #DD #46 #01 |
| BIT 0,(HL) | #CB #46 | BIT 0,(IY-1) | #FD #CB #FF #46 |
| LD A,H | #7C | LD A,IXh | #DD #7C |
| LD H,L | #65 | LD IXh,IXl | #DD #65 |
| LD L,1 | #2E #01 | LD IYl,1 | #FD #2E #01 |
| XOR H | #AC | XOR IXh | #DD #AC |