IDE-Interface Instructions

For MB02

The IDE-Interface offers you a new world in computing with the Spectrum. In this set you bought the hardware-set and the software to run the IDE-Interface with the MB02. Generally the IDE-Interface is also compatible to the Spectrum without MB02, but we give you here the software to run it by using the Operating System of the MB02. This is made by using the Virtual Disk Table made by DRON, the installation software is made by HOOD.

As a special we give you some programs which don´t need MB02 to run:

- IDE-Info: Gives informations about everything connected you your IDE-Interface. There is a readme-File included (Tasword)

- Partition-Info: Gives informations about Partitions.

- MATRIX: New operation system only for IDE, in its first version it doesn´t support any disk-drives

- Acdbrows2: Runs audio-media from CD-ROM, source code for Prometheus

- Vcdbrows2: Runs video-media from CD-ROM, source code for Prometheus

- FDISK: Formats Harddisks, creates partitions.

- CDPD02: Runs normal Audio-CD, source code for Prometheus

This programs are MB02 only:

- HDD_Setup: Helps creating the Virtual Disk Table

- 121_4_benk: Code for changing Operating System of the MB02 for running IDE-Interface

- SEARCH 1.0: Simple program to search trough all Virtual Disks for a file.

# How to build IDE-Interface

To build the IDE-Interface should not be a big problem, it is quite simple.  All parts are included in your set. The following circuits should help you to connect all parts correctly:

Start working

After building the IDE-Interface you have to connect it. The connector for the ZX-Bus is in two parts, one to be soldered to the ZX-Bus, one to the IDE-Interface. For working with the IDE-Interface you should connect both parts together. Please watch out for connecting the right side, as shown in the circuit. So you can always connect or disconnect it (of course only when the computer is switched off).

So everything is soldered and connected. Now you need a IDE-lead to connect an IDE-Device (Harddisk, CD-ROM) to the IDE-Interface and run the IDE-Info (on MB02 type RUN “ideinfo”). There should be now written all details of your connected device. If the program will not find any device connected, there is something wrong. Maybe you connected the IDE-lead in the wrong connection. Turn it around and try again. Nothing will happen with your IDE-Interface if you connected the IDE-lead wrong, you don´t have to worry.

If you build your IDE – Interface correctly, now everything must work. As a next step you should now format your Harddisk. The MB02 access 256 drives (drive 0 to 255). Drive 0 is the tape, drive 1 and 2 are probably your disk drives. So this leaves 253 drives with each 2 MB = 506 MB. It makes no sense to get a bigger harddisk, you can´t use it for now. Future updates might have more possibilities.  Use now the FDISK programm (RUN “fdisk). Best you choose option “o” and create a new partition table. Create one partition as big as your harddisk is but max. 506 MB. Choose Primary Partition and No1. That´s enough, exit with option “w” (write table and exit).

You can now recheck the partition you made by using the Partition – Info Program. Do not choose Option 5 “Create VDT”. This option does not work properly.

For creating the VDT now run the HDD_Setup. It will first search through your system and will find normally drive 1 + 2 and the RAMDISK. Unfortunately it looks like the RAMDISK is lost after you installed the VDT. However, there will be a update with using the RAMDISK but in this version not. So watch out you got no files in your RAMDISK which you need, because they will be erased. What now follows is very easy. The program asks you for BS-DOS-Disk. You have to type in the number for the next drive you want to use. Best thing is normally to create all possible drives, so you have to type “3” as next drives , then 4, 5, 6... not 9 as this is used by RAMDISK (although not working) and if you have a 506 MB partition you can type in all numbers up to 255. If you are tired of this, you can just write the keyword command STOP and continue the program at Line 295. It asks you to write the VDT-Table. Answer with “A” and  it will be saved. The program gives you now some instructions, but forget them, here are better ones and in English:

On your disk you will now find a file **VDT**.  Load in the Program-Code “121_4_benk” in your memory and then add the VDT by loading “** VDT  **” CODE 32771. Now you have everything you need in memory and just have to save it by typing  SAVE “name” CODE 32768, 2000. The VDT is not 2000 Bytes long, it is a little bit shorter but never mind. You now have more memory you ever dreamt of on your Spectrum. Whenever you now want to use the VDT, you have to RUN the program you just saved. The VDT will stay in memory as long as you do not reboot or any other software will kill it. Of course you can also add this option to your autoexe-file on your boot-disk.

Before you can save anything now on your new drives, you have to FORMAT them. Use the normal FF17-Formatter by using the following data:

Drv = drive you want to format

Hdd = 1

Trk = 127

Sec = 8

Use only these datas as they will be compatible to future updates of the software. It is recommended first to format all drives before saving anything. This will take hours, but this is better than to find out a few years later, that you can´t copy from some drives to others. But if you can´t await it, of course you can only create drive 3 and format it for some tests.

The IDE-Interface and harddisk is much faster than normal disks, although it will be even faster with new program version.

Have fun.

If you have any questions, contact  us: sintech@online.de